Safety Training

Manual

for

IUP Police and

Maintenance

Personnel

Prepared by

Dr. John T. Wood

Department of Chemistry

TRAINING MANUAL

(POLICE AND MAINTENANCE)

INTRODUCTION:

We come into contact with chemicals every day.  Some of these might not be recognized as such.  They include things like oven cleaners, drain openers and emergency flares.  These are much more hazardous than many chemicals found in the laboratory.  Other materials found in the home are routinely used in the laboratory.  Sodium bicarbonate (bicarb), sodium chloride (table salt), sucrose (sugar) and sodium hypochlorite (bleach) are examples of these.  Thus, it may be seen that many chemicals are essential and beneficial substances.  However, they must be used properly.  One of the major operating principles in the safe handling of hazardous chemicals is to remember to use common sense in uncommon situations.

I.	LABELS

All chemicals in the College of Natural Science and Mathematics must be 		labeled appropriately.  The label on the chemical will either be one 	generated In House by faculty or student, or it will be one affixed by a 	supplier.

A.	In House labels must contain key signal words on hazardous 			materials.  You should become familiar with the dangers denoted 			by these words:

1.  Corrosive:  A liquid or solid that causes visible destruction or 			irreversible alterations in human skin tissue at the site of contact.  			Common examples:  concentrated acids, concentrated bases.

2.  Poison Class A:  Denotes the most poisonous rating.

3.  Flammable:  A substance capable of being ignited.

4.  Explosive Class A:  Denotes the most dangerous class.

5.  Oxidizer:  This substance initiates or promotes combustion in 			other materials.

6.  Peroxide:   A substance which may decompose by itself to 			release large amounts of energy.  May initiate a reaction in other 			substances.

7.  Lachrymator:  A substance which causes intense irritation of 			mucous membranes and eyes.

8.  Radioactive Material:  Any material which spontaneously emits 		ionizing radiation.

9.  Biohazard:  A viable microorganism, or its toxin, which causes 			or may cause human disease.  Pathogen.

10. Carcinogenic:  Human exposure to this substance may result 			in the development of cancer.

11.  Mutagenic:  Human exposure to this substance may produce 			heritable genetic effects.

12.  Teratogenic:  Human exposure to this substance may produce 		non-heritable birth defects in offspring.

B.	Most of the chemicals obtained from chemical supply houses will 			have affixed the National Fire Protection Association 704 (NFPA 			704) system of protective labeling.

The chemical will have a color coded diamond on the container.  			The diamond is divided into four sections.  Three of the four 				sections have a number in them.  This number indicates the 				severity of the hazard.  The numbers rang from 0 to 4, with 4 the 			most hazardous.

The left section of the diamond is blue in color and informs the user 		of the health hazard of the substance.

The top section of the diamond is red.  This indicates the 				flammability hazard.

The right section of the diamond is yellow and indicates the 				reactivity hazard.

The bottom section of the diamond is white and contains special 			warning messages about the substance.

Examples:	w (react to water)

oxy (oxidizer)

p (polymerization hazard)

HEALTH RATINGS:

4.	Deadly

Materials which on very short exposure could cause death or major 			residual injury even though prompt medical treatment was given.  			Protective clothing and respiratory equipment with independent air 			supply must be used in an area contaminated with this material.  			Caution!!  This material may penetrate rubber clothing.

3.	Extreme Danger

Material which on short exposure could cause serious temporary or 		residual injury even though prompt medical treatment was given.  			Protective clothing and respiratory equipment with an independent 			air supply must be used in an area contaminated with this material.

2.	Hazardous

Material which on intense or continued exposure could cause 			temporary incapacitation or possible residual injury unless prompt 			medical treatment is given.  Protective clothing and respiratory 			equipment with an independent air supply should be used in an 			area contaminated with this material.

1.	Slightly Hazardous

Material which upon exposure would cause irritation but only minor 			residual injury even if no treatment is given.  May require protective 			clothing and approved canister type gas mask in an area 				contaminated with this material.

0.	Non Hazardous

Material which on exposure under fire conditions would offer no 			hazard beyond that of ordinary combustible material.

FLAMMABLE RATINGS:

Susceptibility of materials to burning.

4.	Extremely Flammable

Materials which will rapidly or completely vaporize at atmospheric 			pressure and normal room temperature and which will burn rapidly.  		Materials which have a flash point below 73ºF and a boiling point 			below 100ºF.

3.	Very Flammable

Materials which can be ignited under atmospheric pressure and 			ambient temperature.  Materials which have a flash point below 			100ºF and have a boiling point below 100ºF.  These materials 			usually do not burn explosively.  This class includes pyrophoric 			solids, such as white phosphorus, that spontaneously 	ignite in air.

2.	Flammable

Materials that must be moderately heated or exposed to relatively 			high ambient temperature before ignition can occur.  Materials in 			this rating would not form a hazardous atmosphere with air.  This 			class includes liquids with a flash point above 100ºF, but not 				exceeding 200ºF.

1.	Combustible

Materials that must be preheated before ignition can occur.  				Materials in this rating require considerable preheating before 			ignition and combustion can occur.  Materials have a flash point 			above 200ºF.  These will burn in air when exposed to a temperature 		of 1500ºF for a period of 5 minutes or less.

0.	Non Flammable

Materials will not burn.  Ignition in air will not occur when these are 			exposed to a temperature of 1500ºF for a period of 5 minutes.

REACTIVITY RATINGS:

The ratings are ranked according to ease of initiation of reaction, rate of reaction and quantity of energy released.

4.	Extremely Explosive

These materials are readily capable of detonation or of explosive 			decomposition or reaction at room temperature and atmospheric 			pressure.  They are sensitive to mechanical or localized thermal 			shock at room temperatures and pressure.

3.	Explosive

These materials are capable of detonation or explosive reaction but 		require a strong initiating source or which must be heated under 			confinement before initiation.  This class also includes substances 			which react explosively with water.  These include materials which 			are sensitive to thermal shock or mechanical shock at elevated 			temperatures and pressures.

2.	Extremely Reactive

These materials are normally unstable and readily undergo violent 			chemical change but do not detonate.  These are materials which 			undergo a rapid release of energy at room temperature and 				atmospheric pressure.  It also includes materials which react 			violently with water.

1.	Reactive

These materials are normally stable but can become unstable at 			elevated temperatures and pressures.

0.	Stable

These materials are normally inert even upon exposure to fire.

SPECIFIC HAZARD:

oxy	Oxidizer

This substance initiates or promotes combustion in other materials.

alk	Alkali

This material is caustic.  It will severely damage skin, eyes, etc.

cor	Corrosive

This material will reactive with metal surfaces.  This is also caustic.

w	Use no water

Do not allow this material to come into contact with water.  It will react 	violently with water.

II.	CHEMICAL SPILLS

A.  CHEMICAL SPILLS ON THE BODY

Immediate action must be undertaken when chemicals or reagents are 	spilled on clothing or the skin.  If the substance contacts the clothes, the 	best first action the person can take is to remove the contaminated article 	of clothing.  The area of skin beneath the area of contamination should 	then be flushed with running water.  The rinse should continue for 	approximately 20 minutes.

Caution:  Alkali material will cause much more damage to tissue than an 	acid of comparable concentration.  If the area of contamination is large, 	the person should use the body shower.  While under the stream of water 	from the shower, the person should remove the articles of clothing 	affected.

B.  CHEMICAL SPLASHES IN THE EYE

The major point here:  GET TO THE EYE WASH STATION OR ANY 	SOURCE OF WATER (WHETHER IT IS DRINKING FOUNTAIN OR 	SINK) AND WASH THE EYE THOROUGHLY.  The eye must be flushed 	for at least 20 minutes with flowing water.  The person should then be 	transported immediately to the hospital emergency room.

C.  SPILLS ON THE FLOOR, ETC.

The material should be contained using the Spill Control Kits found in the 	chemical stockroom or the various prep rooms.  If this is not available, the 	absorbent used in the power plant will do.  After the spill is contained, the 	Chemical Hygiene Officer of the Dean of CNSM should be contacted.

III.	ENCOUNTERING AN UNKNOWN LIQUID

If you encounter an unknown liquid spilled in the area you are in, it may be 	of value to determine if it is an organic liquid.  This would help in 	assessing the danger of fire in the area.  In general, organic liquids will 	have an odor like one of the following:

1.  Gasoline, lighter fluid or kerosene.

2.  Fingernail polish remover.

3.  Paint thinner.

4.  Oil base paint.

5.  “Starter” fluid.

6.  Somewhat like ammonia.

The above odors may indicate the presence of a substance with a 	flammability rating of at least 2.  The appropriate precautions should be 	taken.

IV.	MSDS

If needed, a Material Safety Data Sheet (MSDS) can be obtained for materials present in the College of Natural Science and Mathematics.  These can be obtained in the chemical stockroom or from the Campus Safety Office in the Administrative Annex.  Once the MSDS for a particular material is obtained, you should be aware that the MSDS outlines only minimum precautions for the safe handling of a chemical.  It also documents fire and 	explosion hazards, spill or leak procedures, special protection information and special precautions.  The MSDS should also contain first aid procedures.  When an MSDS fails to mention a particular detrimental health effect, it should not be assumed that the substance is hazard free.  	The supplier might not have had access to the most recent test data.