SPILL PREVENTION CONTROL AND COUNTERMEASURE (SPCC) PLAN

INDIANA UNIVERSITY OF PENNSYLVANIA

INDIANA, PENNSYLVANIA

TABLE OF CONTENTS

Page

Spill Prevention Control and Countermeasure Compliance Inspection

Plan Review Page	iv

Professional Engineers Certification	v

Management Approval	vi

1.0	INTRODUCTION

2.0	FACILITY OWNER AND OPERATOR

2.1	Facility Owner, Address and Telephone Number	2-1

2.2	Facility Operator, Address and Telephone Number	2-1

3.0	FACILITY DESCRIPTION

3.1	Physical Location and Layout	3-1

3.2	Description of Operations	3-1

3.2.1	S. W. Jack Cogeneration Plant	3-1

3.2.2	Robertshaw Building Automotive Services	3-2

3.2.3	Stapleton Library	3-2

3.2.4	Gas Wells	3-2

3.3	Facility Drainage	3-3

4.0	SPCC RESPONSE ORGANIZATION AND EMERGENCY NOTIFICATION

4.1	Emergency Classifications	4-1

4.1.1	Workstation Spills	4-1

4.1.2	Minor Emergency	4-2

4.1.3	Major Emergency	4-2

4.2	SPCC Coordinators	4-2

4.2.1	Duties of the SPCC Coordinators	4-3

4.3	Emergency Notifications	4-4

4.3.1	Internal Notifications	4-4

4.3.2	Reportable Quantity Agency Notifications	4-4

4.3.3	Information Exchange and Reporting Forms	4-5

4.4	Emergency Response	4-5

4.5	Outside Emergency Response Personnel	4-7

5.0	SPILL HISTORY

SPILL PREVENTION CONTROL AND COUNTERMEASURE (SPCC) PLAN

INDIANA UNIVERSITY OF PENNSYLVANIA

INDIANA, PENNSYLVANIA

TABLE OF CONTENTS (continued)

Page

6.0	SPCC INVENTORY AND POTENTIAL SPILL PATHWAYS

6.1	SPCC Unit Inventory	6-1

6.1.1	Underground Storage Tanks	6-1

6.1.2	Aboveground Storage Tanks	6-2

6.1.3	Drum and Miscellaneous Petroleum Storage	6-2

6.2	Potential Spill Pathways	6.2

6.2.1	Minor Spill Failures	6-3

6.2.2	Major Spill Failures	6-3

7.0	SPILL/LEAK PREVENTION MEASURES

7.1	Bulk Petroleum Storage Secondary Containment/Drainage Control	7-1

7.1.1	Underground Storage Tanks	7-1

7.1.2	Aboveground Storage Tanks	7-1

7.1.2.1	S. W. Jack Cogeneration Plant	7-1

7.1.2.2	Robertshaw Automotive Services	7-3

7.2	Facility Transfer Operations	7-3

7.2.1	S. W. Jack Cogeneration Plant	7-4

7.2.2	Robertshaw Automotive Services	7-4

7.3	Facility Loading/Unloading Operations	7-4

7.3.1	S. W. Jack Cogeneration Plant	7-5

7.3.2	Robertshaw Automotive Services	7-6

7.4	Material Compatibility	7-6

7.5	Inspection and Monitoring Program	7-7

7.5.1	S. W. Jack Cogeneration Plant	7-8

7.6	Preventive Maintenance	7-8

7.7	Housekeeping		7-8

7.8	Security			7-9

7.8.1	S. W. Jack Cogeneration Plant	7-9

7.8.2	Robertshaw Automotive Services	7-9

7.9	External Factor Planning	7-10

7.10	Personnel Training and Exercises	7-10

7.10.1	Employees	7-10

7.10.2	Local Emergency Response Personnel	7-10

7.10.3	Contractors	7-11

7.11	Spill Control Equipment	7-11

8.0	PLAN DEFICIENCY AND IMPLEMENTATION SCHEDULE

9.0	REFERENCES

SPILL PREVENTION CONTROL AND COUNTERMEASURE (SPCC) PLAN

INDIANA UNIVERSITY OF PENNSYLVANIA

INDIANA, PENNSYLVANIA

TABLE OF CONTENTS (continued)

APPENDIX A – DRAWINGS AND FIGURES

Figure A-1	Site Location Map

Figure A-2	University Layout

Drawing A-3	SPCC Storage Tanks Site Map

Drawing A-4	Storm Lines

Figure A-5	Emergency Discovery Flowchart

Figure A-6	Emergency Source Control/Remediation Procedures

Drawing A-7	Cogeneration Plant Site Map

APPENDIX B – TABLES

Table B-1	Petroleum Product Release Notification Procedures

Table B-2	Emergency Notification List

Table B-3	Spill History

Table B-4	Tank Inventory

Table B-5	Tank Secondary Containment Summary

Table B-6	Spill Control and Emergency Response Equipment

Table B-7	Plan Deficiencies

APPENDIX C – FORMS

Form C-1	Emergency Release Notification Form

APPENDIX D – S. W. JACK COGENERATION PROCEDURES

Emergency Procedure I-7-A:	Lubrication Oil/Fuel Oil/Spill Containment

Emergency Procedure I-3-B:	Diesel Fuel Shutoff to the Co-Generation Engines

Operating Procedure I-19:	Unloading Fuel Oil Tankers-Safety Precautions

SPILL PREVENTION CONTROL AND COUNTERMEASURE (SPCC) PLAN

INDIANA UNIVERSITY OF PENNSYLVANIA

INDIANA, PENNSYLVANIA

SPRILL PREVENTION CONTROL AND COUNTERMEASURE COMPLIANCE

INSPECTION PLAN REVIEW PAGE

In accordance with 40 CFR 112.5 (b), a review and evaluation of this SPCC Plan is conducted at least once every three years.  As a result of this review and evaluation, Indiana University of Pennsylvania will amend the SPCC Plan within six months of the review to include more effective prevention and control technology if:  (1) such technology will significantly reduce the likelihood of a spill event from the facility, and (2) if such technology has been field-demonstrated at the time of the review.  Any amendment to the SPCC Plan shall be certified by a Professional Engineer within six months after a change in the facility design, construction, operation, or maintenance occurs which materially affects the facility’s potential for the discharge of oil into or upon the navigable waters of the United States or adjoining shorelines.

Review Dates					Signature

1.

2.

3.

4.

5.

6.

7.

8.

9.

10.

SPILL PREVENTION CONTROL AND COUNTERMEASURE (SPCC) PLAN

INDIANA UNIVERSITY OF PENNSYLVANIA

INDIANA, PENNSYLVANIA

PROFESSIONAL ENGINEER CERTIFICATION

A certification is required under 40 CFR Section 112.3 (d):

Requirements for Preparation and Implementation of Spill Prevention, Control and Countermeasures Plan:

“No SPCC Plan shall be effective to satisfy the requirements of this part unless it has been reviewed by a Registered Professional Engineer and certified by such Professional Engineer.  By means of this certification the Engineer, having examined the facility and being familiar with the provisions of this part, shall attest that the SPCC Plan has been prepared in accordance with good engineering practices.  Such certification shall in no way relieve the owner or operator of an onshore or offshore facility of his duty to prepare and fully implement such Plan in accordance with Section 112.7 and 112.8, as required by paragraphs (a), (b), and (c) of Section 112.3.”

Therefore, I, George P. Kay, certify that I have inspected the facility and reviewed the SPCC Plan for Indiana University of Pennsylvania and, being familiar with the provisions of 40 CFR 112, attest that the information provided by the facility and contained is herein is, to the best of my knowledge and belief, true, accurate, and complete in accordance with good engineering practices.

Name	Date

George P. Kay, P.E.

License Number PE35251-E

SPILL PREVENTION CONTROL AND COUNTERMEASURE (SPCC) PLAN

INDIANA UNIVERSITY OF PENNSYLVANIA

INDIANA, PENNSYLVANIA

Indiana University of Pennsylvania is committed to the prevention of discharges of oil to navigable waters and the environment and maintains the highest standards for spill prevention control and countermeasures through regular review, updating and implementation of the Spill Prevention Control and Countermeasures Plan for

the University.

Authorized Facility Representative:

Signature and Title